Stainless Steel Pallet Truck

| model |  | EPT22S (Stainless Steel Pallet Truck) |  |
| --- | --- | --- | --- |
| Rated load capacity | kg | 2000 |  |
| Load center distance | mm | 600 |  |
| Front overhang distance | mm | 950 |  |
| wheel base | mm | 1270 |  |
| Self-weight including the storage battery | kg | 145 |  |
| Front wheel dimensions | mm | φ210×75 |  |
| Rear wheel specifications | mm | φ80X70 |  |
| Number of wheels (front/rear; X = drive wheel) | mm | 1x/4 |  |
| Rear wheel track | mm | 390/525 |  |
| Increase height | mm | 110-115 |  |
| Operating handle height: Minimum/Maximum | mm | 585/1250 |  |
| Minimum forklift height | mm | 75/85 |  |
| Overall length | mm | 1620 |  |
| Vehicle length | mm | 470 |  |
| Overall width | mm | 550/685 |  |
| Gantry fork dimensions | mm | 50/160/1150 |  |
| External width of the forks | mm | 550/685 |  |
| Minimum ground clearance | mm | 35 |  |
| Work aisle width: 1000 × 1200 pallet (with a 1200 mm span for fork placement) | Ast(mm) | 2182 |  |
| Work aisle width: 800 × 1200 pallet (the 1200 dimension aligns with the forklift placement); | Ast(mm) | 2052 |  |
| turning radius | (mm) | 1380 |  |
| Operating speed: Full load / Empty load | km/h | 4.2/4.5 |  |
| Enhanced acceleration performance: full load / no-load | m/s | 0.017/0.02 |  |
| Deceleration rate: Full load / No-load | m/s | 0.04/0.04 |  |
| Maximum climbing capability: full load / empty load | % | 6/10 |  |
| Service Brakes |  | electromagnetic braking |  |
| Drive motor power | kw | 0.9 |  |
| Increase motor power | kw | 0.8 |  |
| Battery voltage/capacity | V/ Ah | 12/47×4 (24-hour operation) (Lead-acid battery) | 48/20 (Lithium-ion battery) |
| Battery weight | kg | 5.5 |  |
| Drive Control Method | KWh/h | DC Speed Control |  |
| Noise exposure level at the driver's ear according to EN12053 DB(A) | dBCA) | ≤70 |  |

*Homsun Machinery Equipment Co., Ltd is a professional manufacture specializing in the R &D and producing of electric scissor lift , electric forklift, mobile yard ramp and electric pallet jack  high-quality hardware accessories. Established in 2016, we have over ten years of experience in the industry, dedicated to providing comprehensive solutions of carrying and lifting for all different industries.

*We are the source manufacturer and can also customize various types of forklifts according to customer needs.

*If needed, you can consult customer service.

Official website:https://www.homsunlift.com

WhatsApp/Wechat ID:+8613680275183

Email:sandy519@foxmail.com

Related product recommendations：

Pallet Truck Scale

Stainless steel stacker

Stainless steel forklift

Stainless steel forklift scale